|  | 디지털 일체형 유량 모니터 |  | Model : KM 5 series |  |
| --- | --- | --- | --- | --- |
|  | Digital flow monitor |  |  |  |
|  |  | 본 유량모니터는 유량센서에서 출력되는 펄스 출력을 입력 순간유량 및 적산(누적량), 그래픽 LCD 에서 표시하므로, 사용자가 쉽게 유량 감시를 할 수 있다. 4/20㎃DC, 변환 출력이 가능하며, 적산 펄스를 출력하므로, PLC및 지시계, 카운터에 적용이 가능한 다기능 일체형 유량 모니터이다. |  |  |

Specifications

| Display | 128 * 64 Graphic, backlit LCD |
| --- | --- |
| Display update time | 0.25, 0.5, 1.0, 2.0 Second |
| Rate average time | 0 – 20 Second programmable selection |
| Input power | 24VDC, 3.6V 리튬전지 |
| Date memory | EEPROM stores data for 10 years if power is lost. |
| Output power | DC12V, 50㎃, unregulated (Only “DC” Power Option) |
| Temperature | Operating ( 0 to 70℃ ), Storage ( -20 to 90℃ ) |
| Humidity | 0 – 90 %rh |
| Input | Count input, 5 ㎑ max. |
|  | K-factor range : 0.001 to 99,999,999 Magnetic pick-up, NPN, Reed sw, TTL, Voltage pulse |
|  | 20 Point linearization (Option) |
| Reset | Total ( Menu selection clear ) |
| DC4-20㎃ output | Resolution 12 bit Accuracy : Better than 0.1% Maximum load : Max 750Ω Isolation : Isolated. |
| Pulse output | MOSFET-Photovoltaic Relay 140㎃ Max Pulse width : Off, 10, 100 ㎳ 선택 Frequency : 1 ㎑ max. |

| Material | Cast Aluminum |
| --- | --- |

Dimension

Ordering information

| Model | Code |  |  |  |  | Description |
| --- | --- | --- | --- | --- | --- | --- |
| KM5 |  |  |  |  |  | - Rate & Total Digital Indicator, Ver 1.0 |
| Input type | 0 |  |  |  |  | - Linear |
|  | 1 |  |  |  |  | - 20 Point Linearization |
| Analog Output |  | 0 |  |  |  | - None |
|  |  | 1 |  |  |  | - 4-20㎃ Output ( Not “BA” Power Option ) |
| Relays Output |  |  | 0 |  |  | - None |
|  |  |  | 1 |  |  | - 2 Relay Output ( Only “DC” Power Option ) |
| Communication |  |  |  | 0 |  | - None |
| Power |  |  |  |  | DC | - DC 24V |
|  |  |  |  |  | BA | - 3.6 V 리튬전지 |
|  |  |  |  |  | 2W | - 2-wire loop power |

예: KM5011-0DC ( Linear input, 4-20㎃DC Output, 2 Relay, 24VDC )